Empirical Local Patch

- Make a directory called $(CaMa-Flood-DA).

- You can copy the $(CaMa-Flood-DA) directory directly from the rainbow server. [Location: /cluster/data8/abdul.moiz/20230601_CaMa-DA_shared]. This is preferred since this repository has all the updated codes. Alternatively you can follow steps 3-5 to download the repository from GitHub.

- Set up the CaMa-Flood model in $(CaMa-Flood-DA)/$(CaMa-Flood).

- Create a new directory $(CaMa-Flood-DA)/$(Empirical-Local-Patch).

- Go to $(CaMa-Flood-DA)/$(Empirical-Local-Patch) and download the GitHub repository by running ‘git clone https://github.com/MenakaRevel/Empirical_LocalPatch.git’

- Make shell scripts executable by running ‘chmod +x *.sh’

- Modify params.py for parameters for your Empirical Local Patch setup. For the example of Amazon simulation, the following parameters were used:

| Parameter | Description | Value |
| --- | --- | --- |
| starttime | Starting date for CaMa-Flood simulations | [2000,1,1] |
| endtime | Ending date for CaMa-Flood simulations | [2020,12,31] |
| CaMa_dir | CaMa-Flood model directory | $(CaMa-Flood-DA)/$(CaMa-Flood) |
| out_dir | Empirical Local Patch directory | $(CaMa-Flood-DA)/$(Empirical-Local-Patch) |
| map_name | Name of the CaMa-Flood map used | ‘amz_06min’ |
| input_name | Name of runoff input used | ‘ERA5’ |
| spinup_mode | Switch for spinup | 1 (for doing spinup simulation) 0 (for no doing spinup simulation) |
| spinup_end_year | End year for spinup | 2000 |
| spinup_end_month | End month for spinup | 12 |
| spinup_end_date | End date for spinup | 31 |
| patch_start | Start date for empirical local path | 2000,1,1 |
| patch_end | End date for empirical local path | 2020,12,31 |
| threshold | Weight threshold for empirical local patch | 0.6 |
| cpu_nums | Number of CPUs used for calculation | 40 |
| damrep | Switch for dam representation | 1 (represent dams in local patch) 0 (do not represent dams in local patch) |

- Link input runoff data by running ‘ln -s $(CaMa-Flood-DA)/$(CaMa-Flood)/inp/ $(CaMa-Flood-DA)/$(Empirical-Local-Patch)/CaMa_in’

- Modify s01-CaMa_sim.sh for $(DROFUNIT), $(CROFDIR), $(CROFPRE), $(CROFSUF), $(CDIMINFO) and $(CINPMAT) following CaMa-Flood documentation. For this example, the following values were used:

| Parameter | Descriptions | Values |
| --- | --- | --- |
| DROFUNIT | See CaMa-Flood documentation | "86400" |
| CROFDIR | See CaMa-Flood documentation | "${INBASE}/CaMa_in/"${inputname} |
| CROFPRE | See CaMa-Flood documentation | "Roff____" |
| CROFSUF | See CaMa-Flood documentation | ".sixmin" |
| CDIMINFO | See CaMa-Flood documentation | "${FMAP}/diminfo_amz-06min.txt" |
| CINPMAT | See CaMa-Flood documentation | ${FMAP}/inpmat_amz-06min.bin |

- Before proceeding with the following steps, make sure to modify the PBS settings in each script if you intend to use qsub.

- Execute s01-CaMa_sim.sh by running ‘./s01-CaMa_sim.sh’. This script simply runs the CaMa-Flood model for the specified period.

- Go to src/ and compile all the code. Run ‘make clean’ and then ‘make all’.

- Execute s02-conv_bin2nc.sh by running ‘./s02-conv_bin2nc.sh’. This script converts the binary output from CaMa-Flood to netCDF.

- Execute s03-remove_trend.sh by running ‘./s03-remove_trend.sh’. This script generates the trend removed CaMa-Flood output. For example, ‘rmdtrnd2000-2020.nc’.

- Execute s04-remove_season.sh by running ‘./s04-remove_season.sh’. This script removes seasonality from ‘rmdtrnd2000-2020.nc’ and generates ‘rmdsesn2000-2020.nc’.

- Execute s05-standardize.sh by running ‘./s05-standardize.sh’. This script standardizes ‘rmdsesn2000-2020.nc’ and generates ‘standardized2000-2020.nc’.

- Execute s06-semivariance.sh by running ‘./s06-semivariance.sh’. This script calculates the semivariogram and generates a directory for each grid of CaMa-Flood inside the $(CaMa-Flood-DA)/$(Empirical-Local-Patch)/semivar directory.

- Go to /etc/src and compile all the code by running ‘make clean’ and ‘make all’

- Link GRanD database by running ‘ln -s /home/yamadai/work/data/Dam+Lake/GRanD/inp/GRanD_v1_1_inp.csv dam_inp/damlist.csv’

- Execute s07-weightage.sh by running ‘./s07-weightage.sh’. This will generate spatial weights and gaussian weights in the $(CaMa-Flood-DA)/$(Empirical-Local-Patch)/weightage and $(CaMa-Flood-DA)/$(Empirical-Local-Patch)/gaussian_weight directory, respectively.

- Execute s08-localiz_para.sh by running ‘./s08-localiz_para.sh’. This will generate empirical local patches in the $(CaMa-Flood-DA)/$(Empirical-Local-Patch)/local_patch. In this directory there is a text file for each CaMa-Flood grid. Each text file has the CaMa-Flood coordinates and gaussian weights for the grids defining the local patch.

- You can generate local patches only along the mainstream by executing s09-localizMS_para.sh. [Note: This is not fully tested yet.]

- Compress all the intermediate files by executing s10-archive.sh. Run ‘./s10-archive.sh’. This step is not compulsory and takes quite a lot of time. However, you can save storage space by compressing a large number of files. This also allows for easier transfer of files, since a large number of files in compressed into one file.

- You can plot a time series of the CaMa-Flood simulated water surface elevation, trend removed water surface elevation, seasonality removed water surface elevation and standardized water surface elevation by running ‘python p01_plot_timeseries.py’. Plots are generated for each gauge location in the GRDC database in the region (e.g AMAZONAS). A sample output plot is shown in Figure 1.

Figure 1 - Output of p01_plot_timeseries.py

- To plot a semivariogram, run ‘python p02_plot_semivari.py’. This plot a semivariogram for a river segment. A sample is shown in Figure 2.

Figure 2 - Output of p02_plot_semivari.py

- To plot a spatial map of the spatial weights and empirical local patch run ‘python p03_plot_local_patch.py’. This will generate a spatial weight map and empirical local patch map for various grids corresponding to the location of GRDC gauges. A sample output of spatial weight and local patch is shown in Figure 3a and Figure 3b respectively.

Figure 3a - Output of p03_plot_local_patch.py showing spatial weights

Figure 3b - Output of p03_plot_local_patch.py showing local patch

Ensemble Simulation

- Make a new directory $(CaMa-Flood-DA)/$(Ensemble-Simulation).

- Copy Ensemble Simulation code from the repository. [Location: /cluster/data8/abdul.moiz/20230601_CaMa-DA_shared]

- Link Runoff data from CaMa-Flood directory. Run ‘ln -s $(CaMa-Flood-DA)/$(CaMa-Flood)/inp/ERA5 $(CaMa-Flood-DA)/ $(Ensemble-Simulation)/CaMa_in/ERA5/bin’

- Go to src/ and compile all the codes by running ‘make clean’ and ‘make all’.

- Modify params.py file. The following parameters are used for the sample Amazon simulation

| Parameter | Description | Values |
| --- | --- | --- |
| timestep | Number of seconds in a timestep | 86400 |
| starttime | Start date for ensemble simulation | (2000,1,1) |
| endtime | End date for ensemble simulation | (2020,12,31) |
| start_year | Start year for ensemble simulation | 2000 |
| end_year | End year for ensemble simulation | 2020 |
| org_dir | Ensemble simulation directory | $(CaMa-Flood-DA)/ $(Ensemble-Simulation) |
| CaMa_dir | CaMa-Flood model directory | $(CaMa-Flood-DA)/$(CaMa-Flood) |
| hydro_da_dir | HydroDA directory [For Data Assimilation step] | $(CaMa-Flood-DA)/$(Hydro-DA) |
| runname | Name of input runoff used | ‘ERA5’ |
| input | Name of input runoff used | ‘ERA5’ |
| map_name | Name of CaMa-Flood map used | ‘amz_06min’ |
| ens_mem | Number of ensemble members | 20 |
| expname | Name of experiment | ‘AMZ’ |
| method | Method used to generate ensemble | ‘lognormal’ [‘simple’ and ‘normal’ are other options] |
| distopen | Corrupted runoff's percentage | 0.50 |
| diststd | Noise to make runoff input to scatter ensembles | 0.25 |
| beta | Parameter to generate ensembles | 0.0 |
| alpha | Parameter to generate ensembles | 0.993 |
| E | Error percentage | 0.30 |
| mode | Change depending upon input runoff | 4 |
| run_flag |  | 0 |
| spinup_flag |  | 0 |
| para_nums |  | 6 |
| cpu_nums | Number of CPUs to be used | 20 |

- Make shell scripts executable by running ‘chmod +x *.sh’

- Execute s01-prep_runoff.sh by running ‘./s01-prep_runoff.sh’

- Execute s02-run_CaMa.sh by running ‘./s02-run_CaMa.sh’

- Execute s03-conv_bin2nc.sh by running ‘./s03-conv_bin2nc.sh’

- Execute s04-mk_stat.sh by running ‘./s04-mk_stat.sh’

- Execute s05-copy2HydroDA.sh by running ‘./s05-copy2HydroDA.sh’

AltiMaP

- Make a new directory $(CaMa-Flood-DA)/$(AltiMaP).

- Copy HydroWeb Directory. Include direct download of data.

- Copy AltiMap [s0* and a0* scripts and src]

- Do Chmod +x *.sh to make all scripts executable.

- Modify s01-prep_VSlist.sh for the database that you want to use along with the location of that data directory (for example, HydroWeb). Execute s01-prep_VSlist.sh

- Link glb_06min with full data maps (including 03sec maps) by doing doing ‘ln -s /work/a01/yamadai/work/CaMa_v400/cmf_v400_data/map_full/glb_06min /cluster/data8/abdul.moiz/20230420_CaMa-DA/CaMa-Flood_v4/map/glb_06min’

- Execute s02-allocate_VS.sh

- Modify $obstxt in s03-unreal_obs.sh with the appropriate name of the file (mainly need to change the date). Execute s03-unreal_obs.sh

HydroDA

- Make a new directory $(CaMa-Flood-DA)/$(HydroDA). If you have already run the script $(CaMa-Flood-DA)/$(Ensemble-Simulation)/s05-copy2HydroDA.sh, then this directory should already be created.

- Running $(CaMa-Flood-DA)/$(Ensemble-Simulation)/s05-copy2HydroDA.sh previously copies the mean_sfcelv and std_sfcelv in the $(CaMa-Flood-DA)/$(HydroDA)/dat directory.

- Running $(CaMa-Flood-DA)/$(AltiMaP)/copy2HydroDA_org.py copies the list of all the virtual stations in the target region to $(CaMa-Flood-DA)/$(HydroDA)/dat/HydroWeb_alloc_amz_06min_org.txt

- Modify $(CaMa-Flood-DA)/$(HydroDA)/etc/prep_obs.py with the following parameters for the case of this Amazon simulation:

| Parameter | Description | Value |
| --- | --- | --- |
| starttime() | Start time for including satellite observations | 2014,1,1 |
| endtime() | Time upto which satellite observations will be included | 2015,12,31 |
| obs_name() | Name of the satellite observation dataset | “HydroWeb” |
| CaMa_dir() | Path to the CaMa-Flood model directory | $(CaMa-Flood-DA)/$(CaMa-Flood) |
| mapname() | Name of CaMa-Flood map used | “amz_06min” |

- Execute prep_obs.py by running ‘python prep_obs.py’. This creates a series of text files in $(CaMa-Flood-DA)/$(HydroDA)/obs for each day. Each text file has the ix, iy, value of satellite observation, mean, standard deviation and the name of the satellite observation respectively.

- Go to $(CaMa-Flood-DA)/$(HydroDA)/gosh and compile all the codes for HydroDA by running ‘sh compile.sh yes’

- Modify the parameters for HydroDA $(CaMa-Flood-DA)/$(HydroDA)/gosh/params_real.py with the following values for the case of the Amazon simulation:

| Parameter | Description | Value |
| --- | --- | --- |
| mode() | changes configuration depending upon the runoff dataset used | 5 (for ERA5) |
| conflag() | Flag for observation conversion | 3 (for Normalized Values) |
| mapname() | Name of CaMa-Flood map used | “amz_06min” |
| CaMa_dir() | Path to the CaMa-Flood model directory | $(CaMa-Flood-DA)/$(CaMa-Flood) |
| assimS() | South bound for the region to be assimilated | -20 (degrees) |
| assimN() | North bound for the region to be assimilated | 5 (degrees) |
| assimW() | West bound for the region to be assimilated | -80 (degrees) |
| assimE() | East bound for the region to be assimilated | -45 (degrees) |
| patch_size() | Size of the patch. In case if 100 is specified then empirical local patch will be used | 100 |
| DA_dir() | HydroDA Directory | $(CaMa-Flood-DA)/$(HydroDA) |
| patch_dir() | Directory for empirical local patches | $(CaMa-Flood-DA)/ )/$(Empirical-Local-Patch)/local_patch |
| patch_name() | Name of the empirical local patch directory | 'amz_06min_ERA5_60' |
| patch_id() | ID of the empirical local patch (Same as the threshold, but a string) | “0.60” |
| thersold() | threshold for empirical local patch | 0.60 |
| initial_infl() | initial inflation parameter | 1.08 |
| rho() | inflation parameter | 1.08 |
| sigma_b() | background variance of inflation for adaptive inflation | 0.04 |
| ens_mem(mode=mode()) | number of ensemble members | 20 |
| timestep() | outer time step in seconds | 86400 |
| starttime() | start time for data assimilation | (2014,1,1) |
| endtime() | end time for data assimilation | (2015,1,1) |
| spinup_mode() | decides which experiments should the spinup simulation be done for | 0 (do spinup simulation for both (corrupted/open and true) simulation |
| spinup_end_year() | ending year for spinup simulation | 2013 |
| spinup_end_month() | ending month for spinup simulation | 12 |
| spinup_end_date() | ending date for the spinup simulation | 31 |
| runoff_dir() | Directory for runoff forcings | $(CaMa-Flood-DA)/$(Ensemble-Simulation)/CaMa_in/ERA5 |
| runname(num=mode()) | Name of runoff forcing dataset | “ERA5” |
| input(num=mode()) | Name of runoff forcing dataset | “ERA5” |
| run_flag() | decides which assimilation experiments to run | 0 (run all simulations) |
| CaMa_dir() | CaMa-Flood Model directory | $(CaMa-Flood-DA)/$(CaMa-Flood) |
| obs_name() | Name of the observation dataset to be assimilated | “HydroWeb” |
| HydroWeb_dir() | Location of satellite observation directory | $(CaMa-Flood-DA)/HydroWeb |
| obs_dir() | Location of processed satellite observations | $(CaMa-Flood-DA)/ $(HydroDA)/obs/HydroWeb |
| obs_list() | Location of text file describing list of the satellite observations | DA_dir() + "/dat/HydroWeb_alloc_" + mapname() + "_org.txt" |
| stat_name(cal=calibrate()) | Name of sfcelv statistics file | "sfcelv_20_ERA5_amz_06min_2000-2020" |
| para_nums() | Number of parallel model runs | 20 |

- Modify $(CaMa-Flood-DA)/$(HydroDA)/gosh/run_mool.sh specifying HydroDA=$(CaMa-Flood-DA)/ $(HydroDA) and HydroDAout=$(CaMa-Flood-DA)/ $(HydroDA). Also specify the name of the experiment by setting EXP=’test_wse’ (can be any name).

- Execute run_mool.sh by running ‘./run_mool.sh’. It is preferable to use qsub here if you are using the rainbow server.

- Outputs of the data assimilation experiments are generated in $(CaMa-Flood-DA)/$(HydroDA)/out/$(EXP)/assim_out/.

- A timeseries of the discharge output can be plotted by running p01_plot_timeseries.py in $(CaMa-Flood-DA)/$(HydroDA)/img.